生态环境 2008, 17(3): 923-925                                                           http://www.jeesci.com

Ecology and Environment                                                                E-mail: editor@jeesci.com
924                                                                   生态环境  第17卷第3期（2008年5月）
杨旭等：表流湿地土壤因素对微污染水净化作用                                                              925

表流湿地土壤因素对微污染水净化作用

杨旭，于水利*，修春海，张洪洋，严晓菊，王冬光
哈尔滨工业大学市政环境工程学院，黑龙江 哈尔滨 150090
摘要：为了研究人工湿地土壤在处理微污染原水的作用，在济南玉清湖水库建立土壤空白床进行实验研究。结果表明，在排除了植物因子的前提下，人工湿地的土壤-微生物系统对微污染原水中COD平均去除率达到46.67%、对TN的平均去除率为34.86%、对NH4+-N的平均去除率分别达到39.83、对TP的平均去除率达到45.12%。系统不同单位区段对微污染原水中的COD、TN、NH4+-N、TP去除率存在差异，在第一个单位区段的处理效率最高，分别为21.5%、18.32%、19.85%和16.64%，远大于第二、三、四区段。另外，土壤吸收附磷的过程中存在着积累现象。而且，系统处理效率受温度影响显著，随着温度的降低，污染物去除率也下降。
关键词：土壤-微生物系统；微污染原水；净化效率
中图分类号：TU991.2         文献标识码：A        文章编号：1672-2175（2008）03-0923-03

济南市饮用水大部分以黄河水为水源。由于近年来黄河流域工农业发展迅速，排污量大，导致黄河水质不断恶化，黄河水作为饮用水源的水质状况不容乐观。济南市自来水厂现有工艺难以去除微污染原水中的COD，TN、TP等污染物，为此需要对原水进行预处理。

人工湿地作为一种低成本、低能耗、低技术要求的水处理方法，近年来，其应用的领域不断拓宽，已成功地用于生活污水[1][2]、工业废水[3] [4]、城市暴雨径流[5] [6]、农业废水的处理与管理[7] [8]和湖泊污染防治[9-11]，故拟采用该方法进行原水预处理。目前在人工湿地系统的机理性研究往往偏于生物因子在污水处理过程中的作用，而对非生物因子，特别是作为湿地系统中植物和微生物生长依托的土壤研究相对较少[12]，为了探索湿地土壤因素对微污染水净化作用，在济南市玉清湖水库二号泵站院内，建立了湿地土壤-微生物系统，进行了试验研究。

1  材料和方法
1.1  表流湿地土壤-微生物系统构造
该湿地净尺寸为16 m×5.5 m×0.8 m，平均水深0.20 m，沿水流方向选取4 m、8 m、12 m、16 m处为取样点。湿地床体底面以混凝土浇注，周边砖砌砂浆抹面。布水、出水均采用三角堰，用以均匀布水。布水和出水各设置一个集水池。系统内土壤于五月下旬从当地生长野生芦苇的土壤挖取添入。
1.2  系统运行情况

系统于2006年5月末开始运行，7月进入稳定期，8月3日开始进行检测，12月8日结束。其间经历了夏、秋、冬三个季节，室外气温由高逐渐转低，处理效果由好到差。运行期间，表流湿地系统始终按设计负荷2.70 m/d运行，水力停留时间6 d.

1.3  检测方法
试验所用的进水样取自湿地进集水池，出水样取自各个出水口，每次取样都在上午9点，取样测试频率为6～8次/月。水温：温度计；pH：玻璃电极法；COD：快速法；TN：碱性过硫酸钾消解分光光度法；TP：钼酸盐分光光度法；NH4+-N：纳氏试剂分光光度法；NO2--N：重氮化偶合分光光度法；NO3--N：紫外分光光度法。土壤中总氮、总磷含量测定采用标准开氏法和高氯酸消化法测定[13]。
2  结果与讨论

2.1  总的处理效果

如表1所示：在6个多月的运行监测期间，系统对水中的COD、TP平均去除率在45%以上，对TN、NH4+-N、NO3--N的平均去除率接近40%，TN的平均去除率接近35%。根据地表水环境质量标准GB3838—2002，通过系统净化作用，水中COD由Ⅳ类变为Ⅱ类，TP由Ⅲ类变为Ⅱ类，TN由远高于Ⅴ类变为接近Ⅴ类，NH4+-N由Ⅲ类变为Ⅱ类。
2.2  不同单位区段的处理效果
为了更好说明系统中各区段对污染物的去除效果，计算了各区段平均处理效率，如图1—4所示。在图1—4中，横坐标1、2、3、4表示第一、二、三、四的4 m段。TP区间平均去除率分别为16.64%、15.5%、7.41%、5.57%，COD的区间平均去除率分别为21.5%、11.2%、9.5%、4.47%，TN的区间平均去除率分别为18.32%、7.5%、5.3%、3.74%，NH4+-N区间平均去除率分别为19.85%、8.7%、6.8%、4.48%。通过图1—4可以看出污染物去除作用主要发生在系统前半部分。

表1  系统出入水主要特征及总去除率
Table 1  The characteristic of inflow and outflow and the removal rate of nutrition
	项目
	ρ(进水)/(mg·L-1)
	
	ρ(出水)/(mg·L-1)
	去除率/%

	
	最小值.
	最大值
	平均值±标准差
	最小值.
	最大值
	平均值±标准差
	

	COD
	10.90
	44.30
	23.80±9.57
	5.56
	18.61
	11.90±3.24
	46.67

	TP
	0.0047
	0.4202
	0.1006±0.1287
	0.0026
	0.2185
	0.0530±0.0656
	45.12

	TN
	1.47
	4.61
	3.26±0.88
	0.85
	3.20
	2.14±0.69
	34.86

	NH4+-N
	0.0370
	1.1700
	0.5136±0.2683
	0.0178
	0.7956
	0.3260±0.2031
	39.83

	NO3--N
	0.03
	3.96
	2.99±0.81
	0.02
	2.34
	1.83±0.53
	39.36

	NO2--N
	0.0140
	0.9800
	0.0722±0.1941
	0.0092
	0.7056
	0.0476±0.1405
	39.17


2.3  不同实验阶段土壤的TN、TP含量变化


[image: image1.emf]0

5

10

15

20

4 8 12 16

取样口位置

/m

去除率

/%


图1  TP各区段平均去除率
Fig. 1  The average Removal rate of TP in different units

[image: image2.emf]0

5

10

15

20

25

4 8 12 16

取样口位置

/m

去除率

/%


图2  COD各区段平均去除率
Fig. 2  The average Removal rate of  COD in different units

[image: image3.emf]0

5

10

15

20

25

4 8 12 16

取样口位置

/m

去除率

/%


图3  NH4+-N各区段平均去除率
Fig. 3  The average Removal rate of NH4+-Nin different units

[image: image4.emf]0

5

10

15

20

4 8 12 16

取样口位置

/m

去除率

/%


图4  TN各区段平均去除率
Fig. 4  The average Removal rate of TN in different units
在实验初期、中期和末期，分别又对系统中不同区段土壤的TN、TP进行测定[14]，其结果如表2、表3。由表2、表3可以看出，土壤中总氮含量各区段的增加是同步的。通过鉴定表明：存在大量的硝化细菌和反硝化细菌，这有利于氮的去除。在起始区段，土壤总磷含量从实验早期到中期增加明显，到末期则无明显的增加。而其后的第二区段、第三区段和第四区段的总磷含量在实验后期的增加明显。这表明土壤对总磷的吸附有一定的饱和度。土壤中的总磷含量从起始区段到第四区段依次增加，起始区段首先达到饱和。

表3  土壤的TP含量的变化
Table 3  TP concentration of soil               %
	实验阶段
	取样位置

	
	4 m
	8 m
	12 m
	16 m

	实验早期
	0.046
	0.045
	0.048
	0.047

	实验中期
	0.121
	0.87
	0.064
	0.56

	实验末期
	0.135
	0.119
	0.093
	0.074


表2  土壤的TN含量的变化
Table 2  TN concentration of soil              %
	实验阶段
	取样位置

	
	4 m
	8 m
	12 m
	16 m

	实验早期
	0.071
	0.072
	0.069
	0.071

	实验中期
	0.121
	0.113
	0.108
	0.095

	实验末期
	0.152
	0.135
	0.128
	0.118


2.4  温度对去除效果影响

土壤-微生物系统中微生物作用很重要，而任何一种微生物都有一个最适生长温度，在一定的温度范围内，随着温度的上升，该微生物生长速率加快，其相应对污染物的去除速率也加快。温度影响有机污染物生物降解的原因除了改变微生物的代谢速率外，还能影响有机物的溶解度，所以在实际处理中，控制微生物的最适生长温度才能更好地提高其修复效能。   

系统整个运行期间，水温在0~32 ℃之间变化，由图5可见系统对水中污染物去除作用受温度影响显著，随着温度的降低，污染物去除率也下降。


[image: image5.emf]0

10

20

30

40

50

60

70

80

0.3 13 15 1924.827.829.134.2

t/oC

去除率

/%

a b c d


图5  温度对污染物去除率的影响
Fig. 5  The impact of temperature on removal rate 
for (a) COD，(b)NH4+-N，(c) TN，(d) TP
3  结论

（1）不考虑植物因素的湿地系统对微污染水仍具有较高的处理能力，说明土壤在处理水中污染物过程中发挥着重要作用。
（2）微污染水流经整个系统，在不同区段的处理效果是不同的，处理效率随流程的增加而下降，系统的起始区段具有最大处理效率。
（3）系统运行过程中存在着磷的积累，积累现象从系统的起始区段开始逐渐向后续区段转移。
（4）系统处理效率受温度影响显著，随着温度的降低，污染物去除率也下降。
参考文献：

[1]  Winthrop C A, Paul B H, Joel A B, et al. Temperature and wetland plant species effects on wastewater and root zone oxidation[J]. Journal of Environmental Quality, 2002, 31: 1010-1016.

[2]  Korkusuz E A, Beklioglu M, Demirer N G. Comparison of the treatment performances of blast furnace slag-based and gravel-based vertical flow wetlands operated identically for domestic wastewater treatment in Turkey[J]. Ecological Engineering, 2005, 24 (3):185-198.

[3]  Anne L S, Cynthia A. Design and hadrulic performance of a constructed wetland treating oil refinery wastewater[J]. Water Science and Technology, 1999, 40 (3): 301-307.

[4]  Margaret G. Nutrient content of wetland plants in constructed wetlands receiving municipal effluent in tropical Australia[J]. Water Science and Technology, 1997, 35(5):135-142.

[5]  Ji G D, Sun T H, Zh Q X, et al. Constructed subsurface flow wetland for treating heavy oil-produced water of the Liaohe Oilfield in China[J]. Ecological Engineering, 2002, 18: 459-465.

[6]  Andrew M R, Richard S I. Development of vegetation in a constructed wetland receiving irrigation return flows[J]. Agriculture, Ecosystems & Environment, 2007, 121:401-406.

[7]  Scholes L, Shutes R B E，Revitt D M，et al. The treatment of metals in urban runoff by constructed wetlands[J]. Science of the Total Environment, 1998, 214:211-219.

[8]  Sakadevan K, Bovar H J. Nutrient removal mechanisms in constructed wetlands and sustainable water management[J]. Water Science and Technology, 1999, 40(2):121-128.

[9]  Kem J, Idler C. Treatment of domestic and agricultural wastewater by reed bed systems[J]. Ecological Engineering, 1999, 12:13-25.
[10] Sakadevan K, Bovar H J. Nutrient removal mechanisms in constructed wetlands and sustainable water management[J]. Water Science and Technology, 40(2): 121-28.
[11] Vymazal J. Removal of nutrients in various types of constructed wetlands[J]. Science of the Total Environment, 2007, 380(1):48-65.

[12] Andrew M. Ray，Richard S. Inouye. Development of vegetation in a constructed wetlandreceiving irrigation return flows[J]. Agriculture Ecosystem Environment，2007，121: 401-406.
[13] 鲍士旦. 土壤农化分析[M]. 北京: 中国农业出版社，1999: 39-76.
Bao Shidan. Soil of Agricultural Chemistry Analysis[M]. Chinese Agricultural publishing house，1999：39-76.
[14] R Corstanje, K R Reddy, J P Prenger, et al. Soil microbial eco-physiological response to nutrient enrichment in a sub-tropical wetland[J]. Ecological Indicators, 2007, 7: 277-289.

The role of wetland soil on the purification of the micro-polluted raw water
Yang Xu, Yu Shuili, Xiu Chunhai, Zhang Hongyang, Yan Xiaoju, Wang Dongguang
1. School of Municipal and Environmental Engineering, Harbin Institute of Technology, Harbin 150090, China
Abstract: In order to study the role of wetland soil on the purifying the micro-polluted water, soil-bacteria system is built to study experimentally in Yuqing Lake in Jinan. The removal process of a simulation wetland was studied in the absence of aquatic plants. The results show the average removal rates of COD, TN, NH4+-N and TP were 46.67%, 34.86%, 39.83% and 45.12%，respectively. The 16 m long system was divided into four equal units along the direction of inflow. The removal of COD, TN, NH4+-N and TP mainly occurred in the first unit，with a removal of 21.5%, 13.5%, 19.85 and 16.64%, respectively. This efficiency was much greater than those in the subsequent three units. In addition, the accumulation of P was found in soil and TP of soil would reach a peak. Moreover, the removal of contamination in water is influenced obviously by the system under the temperature. As the temperature goes down, the removal rate of contamination is eliminated rapidly.

Key words: soil-bacteria system; micro-polluted raw water; treatment effect 
基金项目：国家高技术研究发展计划（863）项目(2006AA06Z303)
作者简介：杨旭（1973－），男，工程师，博士研究生，研究方向为饮用水处理。E-mail: yangxu2005gd@163.com
          *通讯作者：于水利，教授
收稿日期：2007-11-16(

_1274181153.xls
Chart2

		4

		8

		12

		16



去除率

取样口位置/m

去除率/%

21.5

11.2

9.5

4.47



Sheet1

						cod

		日期		进水		出水		差值		去除率

		38,932		28.5		8.835		19.665		69

		38,937		39.8		14.328		25.472		64

		38,941		35.5		13.49		22.01		62

		38,944		44.3		18.606		25.694		58

		38,954		38.6		15.826		22.774		59

		38,977		33.5		14.405		19.095		57

		38,980		31.8		15.264		16.536		52

		38,983		25.6		11.776		13.824		54

		38,986		25.7		12.593		13.107		51

		38,988		24.9		12.948		11.952		48

		38,991		21.3		10.224		11.076		52

		38,997		12.1		6.413		5.687		47

		39,002		22.5		12.15		10.35		46

		39,006		27.8		15.846		11.954		43

		39,009		10.9		5.559		5.341		49

		39,013		16.6		8.964		7.636		46

		39,019		13.6		7.616		5.984		44

		39,024		22.9		13.511		9.389		41

		39,031		15.2		9.424		5.776		38

		39,038		14.7		9.702		4.998		34

		39,045		18.2		11.83		6.37		35

		39,049		19		15.01		3.99		21

		39,053		12.9		9.546		3.354		26

		39,059		15.4		11.704		3.696		24

				23.8041666667		11.89875				46.6666666667

		日期		进水





Sheet2

		

						cod

		日期		进水		出水		进水		去除量				去除率

		38,932		28.5		8.835		28.5		19.665				69

		38,937		39.8		14.328		39.8		25.472				64

		38,941		35.5		13.49		35.5		22.01				62

		38,944		44.3		18.606		44.3		25.694				58

		38,954		38.6		15.826		38.6		22.774				59

		38,977		33.5		14.405		33.5		19.095				57

		38,980		31.8		15.264		31.8		16.536				52

		38,983		25.6		11.776		25.6		13.824				54

		38,986		25.7		12.593		25.7		13.107				51

		38,988		24.9		12.948		24.9		11.952				48

		38,991		21.3		10.224		21.3		11.076				52

		38,997		12.1		6.413		12.1		5.687				47

		39,002		22.5		12.15		22.5		10.35				46

		39,006		27.8		15.846		27.8		11.954				43

		39,009		10.9		5.559		10.9		5.341				49

		39,013		16.6		8.964		16.6		7.636				46

		39,019		13.6		7.616		13.6		5.984				44

		39,024		22.9		13.511		22.9		9.389				41

		39,031		15.2		9.424		15.2		5.776				38

		39,038		14.7		9.702		14.7		4.998				34

		39,045		18.2		11.83		18.2		6.37				35

		39,049		19		15.01		19		3.99				21

		39,053		12.9		9.546		12.9		3.354				26

		39,059		15.4		11.704		15.4		3.696				24

				23.8041666667		11.89875								46.6666666667





Sheet2

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0



去除量

COD in influents/mg.L-1

COD removal/mg.L-1

y = 0.7267x - 5.3941

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0



Sheet3

						COD不同区段去除率的变化

		4		8		12		16

		21.5		11.2		9.5		4.47

				去除率

		4		21.5

		8		11.2

		12		9.5

		16		4.47





Sheet3

		0

		0

		0

		0



去除率

取样口位置/m

去除率/%

0

0

0

0




_1274181625.xls
Chart2

		4

		8

		12

		16



去除率

取样口位置/m

去除率/%

18.32

7.5

5.3

3.74



Sheet1

						TN

		日期		进水		出水		去除量		去除率

		38,932		3.04		1.5504		1.4896		49

		38,941		2.638		1.39814		1.23986		47

		38,944		2.79072		1.4232672		1.3674528		49

		38,954		1.73		0.8477		0.8823		51

		38,980		2.66		1.5162		1.1438		43

		38,983		2.86		1.6874		1.1726		41

		38,986		3.46		2.076		1.384		40

		38,988		4.61		2.9043		1.7057		37

		38,991		3.99		2.6733		1.3167		33

		38,997		4.016		2.6104		1.4056		35

		39,006		4.5261		2.987226		1.538874		34

		39,009		3.643		2.47724		1.16576		32

		39,013		4.5137		3.204727		1.308973		29

		39,019		4.0535		2.796915		1.256585		31

		39,024		1.4664		1.041144		0.425256		29

		39,031		3.67		2.6791		0.9909		27

		39,038		3.2		2.272		0.928		29

		39,045		3.08		2.2792		0.8008		26

		39,049		2.92		2.2192		0.7008		24

		39,053		2.05		1.5375		0.5125		25

		39,059		3.49		2.7571		0.7329		21

				3.2574961905		2.1399266286				34.8571428571

		日期		进水





Sheet2

		

						TN

		日期		进水		出水		进水		去除量		去除率

		38,932		3.04		1.5504		3.04		1.4896		49

		38,941		2.638		1.39814		2.638		1.23986		47

		38,944		2.79072		1.4232672		2.79072		1.3674528		49

		38,954		1.73		0.8477		1.73		0.8823		51

		38,980		2.66		1.5162		2.66		1.1438		43

		38,983		2.86		1.6874		2.86		1.1726		41

		38,986		3.46		2.076		3.46		1.384		40

		38,988		4.61		2.9043		4.61		1.7057		37

		38,991		3.99		2.6733		3.99		1.3167		33

		38,997		4.016		2.6104		4.016		1.4056		35

		39,006		4.5261		2.987226		4.5261		1.538874		34

		39,009		3.643		2.47724		3.643		1.16576		32

		39,013		4.5137		3.204727		4.5137		1.308973		29

		39,019		4.0535		2.796915		4.0535		1.256585		31

		39,024		1.4664		1.041144		1.4664		0.425256		29

		39,031		3.67		2.6791		3.67		0.9909		27

		39,038		3.2		2.272		3.2		0.928		29

		39,045		3.08		2.2792		3.08		0.8008		26

		39,049		2.92		2.2192		2.92		0.7008		24

		39,053		2.05		1.5375		2.05		0.5125		25

		39,059		3.49		2.7571		3.49		0.7329		21

				3.2574961905		2.1399266286						34.8571428571

		日期		进水





Sheet2

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0



去除量

TN in influents/mg.L-1

TN removal/mg.L-1

y = 0.2714x + 0.2336

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0



Sheet3

		

				去除率

		4		18.32

		8		7.5

		12		5.3

		16		3.74





Sheet3

		0

		0

		0

		0



去除率

取样口位置/m

去除率/%

0

0

0

0




_1274181889.xls
Chart1

		0.3		0.3		0.3		0.3

		8.1		8.1		8.1		8.1

		12.4		12.4		12.4		12.4

		13		13		13		13

		13.1		13.1		13.1		13.1

		15		15		15		15

		15		15		15		15

		15.8		15.8		15.8		15.8

		15.9		15.9		15.9		15.9

		19		19		19		19

		20		20		20		20

		23.5		23.5		23.5		23.5

		24.8		24.8		24.8		24.8

		25.6		25.6		25.6		25.6

		26		26		26		26

		27.8		27.8		27.8		27.8

		27.9		27.9		27.9		27.9

		28		28		28		28

		29.1		29.1		29.1		29.1

		31.9		31.9		31.9		31.9

		32.5		32.5		32.5		32.5

		34.2		34.2		34.2		34.2



a

b

c

d

t/oC

去除率/%

24

25

20

26

26

23

21

31

41

32

26

32

38

34

24

29

44

36

29

26

46

33

27

31

34

32

25

34

43

38

31

39

49

37

29

33

46

41

29

40

47

39

32

42

52

40

34

44

54

43

37

48

52

47

40

51

62

52

49

55

59

55

43

53

69

51

49

57

57

49

41

49

51

46

33

49

48

41

35

47

58

53

51

50

64

53

47

53



Sheet1

		

				温度

						cod		NH-N		TN		TP

		38,932		27.9		69		51		49		57

		38,937		34.2		64		53		47		53

		38,941		26		62		52		49		55

		38,944		32.5		58		53		51		50

		38,954		27.8		59		55		43		53

		38,977		28		57		49		41		49

		38,980		25.6		52		47		40		51

		38,983		24.8		54		43		37		48

		38,986		29.1		51		46		33		49

		38,988		31.9		48		41		35		47

		38,991		23.5		52		40		34		44

		38,997		20		47		39		32		42

		39,002		19		46		41		29		40

		39,006		15.8		43		38		31		39

		39,009		15.9		49		37		29		33

		39,013		15		46		33		27		31

		39,019		13.1		44		36		29		26

		39,024		12.4		41		32		26		32

		39,031		13		38		34		24		29

		39,038		15		34		32		25		34

		39,053		8.1		26		23		21		31

		39,059		0.3		24		25		20		26





Sheet2

		

						COD		NH4+-N		TN		TP

						a		b		c		d

		39,059		0.3		24		25		20		26

		39,053		8.1		26		23		21		31

		39,024		12.4		41		32		26		32

		39,031		13		38		34		24		29

		39,019		13.1		44		36		29		26

		39,013		15		46		33		27		31

		39,038		15		34		32		25		34

		39,006		15.8		43		38		31		39

		39,009		15.9		49		37		29		33

		39,002		19		46		41		29		40

		38,997		20		47		39		32		42

		38,991		23.5		52		40		34		44

		38,983		24.8		54		43		37		48

		38,980		25.6		52		47		40		51

		38,941		26		62		52		49		55

		38,954		27.8		59		55		43		53

		38,932		27.9		69		51		49		57

		38,977		28		57		49		41		49

		38,986		29.1		51		46		33		49

		38,988		31.9		48		41		35		47

		38,944		32.5		58		53		51		50

		38,937		34.2		64		53		47		53





Sheet2

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0



a

b

c

d

t/oC

去除率/%

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0



Sheet3

		






_1274181379.xls
Chart1

		4

		8

		12

		16



去除率

取样口位置/m

去除率/%

19.85

8.7

6.8

4.48



Sheet1

						NH-N

		日期		进水		出水		去除量		去除率

		38,932		0.28		0.1372		0.1428		51

		38,937		0.398		0.18706		0.21094		53

		38,941		0.037		0.01776		0.01924		52

		38,944		0.039		0.01833		0.02067		53

		38,954		0.46		0.207		0.253		55

		38,977		0.54		0.2754		0.2646		49

		38,980		0.47		0.2491		0.2209		47

		38,983		0.31		0.1767		0.1333		43

		38,986		0.42		0.2268		0.1932		46

		38,988		0.5		0.295		0.205		41

		38,991		0.23		0.138		0.092		40

		38,997		0.529		0.32269		0.20631		39

		39,002		0.335		0.19765		0.13735		41

		39,006		0.6142		0.380804		0.233396		38

		39,009		0.751		0.47313		0.27787		37

		39,013		0.4433		0.297011		0.146289		33

		39,019		0.3692		0.236288		0.132912		36

		39,024		0.9105		0.61914		0.29136		32

		39,031		0.48		0.3168		0.1632		34

		39,038		1.17		0.7956		0.3744		32

		39,045		0.59		0.4189		0.1711		29

		39,049		0.81		0.5913		0.2187		27

		39,053		0.78		0.6006		0.1794		23

		39,059		0.86		0.645		0.215		25

				0.5135916667		0.3259692917				39.8333333333

		日期		进水





Sheet2

		

						NH-N

		日期		进水		出水		进水		去除量		去除率

		38,932		0.28		0.1372		0.28		0.1428		51

		38,937		0.398		0.18706		0.398		0.21094		53

		38,941		0.037		0.01776		0.037		0.01924		52

		38,944		0.039		0.01833		0.039		0.02067		53

		38,954		0.46		0.207		0.46		0.253		55

		38,977		0.54		0.2754		0.54		0.2646		49

		38,980		0.47		0.2491		0.47		0.2209		47

		38,983		0.31		0.1767		0.31		0.1333		43

		38,986		0.42		0.2268		0.42		0.1932		46

		38,988		0.5		0.295		0.5		0.205		41

		38,991		0.23		0.138		0.23		0.092		40

		38,997		0.529		0.32269		0.529		0.20631		39

		39,002		0.335		0.19765		0.335		0.13735		41

		39,006		0.6142		0.380804		0.6142		0.233396		38

		39,009		0.751		0.47313		0.751		0.27787		37

		39,013		0.4433		0.297011		0.4433		0.146289		33

		39,019		0.3692		0.236288		0.3692		0.132912		36

		39,024		0.9105		0.61914		0.9105		0.29136		32

		39,031		0.48		0.3168		0.48		0.1632		34

		39,038		1.17		0.7956		1.17		0.3744		32

		39,045		0.59		0.4189		0.59		0.1711		29

		39,049		0.81		0.5913		0.81		0.2187		27

		39,053		0.78		0.6006		0.78		0.1794		23

		39,059		0.86		0.645		0.86		0.215		25

				0.5135916667		0.3259692917		0.5135916667				39.8333333333

		日期		进水





Sheet2

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0



去除量

NH4+-N in influents/mg.L-1

NH4+-N removal/mg.L-1

y = 0.2584x + 0.0549

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0



Sheet3

		

								NH不同区段去除率的变化

				4		8		12		16

				19.85		8.7		6.8		4.48

						去除率

				4		19.85

				8		8.7

				12		6.8

				16		4.48





Sheet3

		0

		0

		0

		0



去除率

取样口位置/m

去除率/%

0

0

0

0




_1274180972.xls
Chart2

		4

		8

		12

		16



去除率

取样口位置/m

去除率/%

16.64

15.5

7.41

5.57



Sheet1

						TP

		日期		进水		出水		去除量		去除率

		38,944		0.282		0.12126		0.16074		57

		38,954		0.014		0.00658		0.00742		53

		38,980		0.026		0.0117		0.0143		55

		38,983		0.041		0.0205		0.0205		50

		38,986		0.014		0.00658		0.00742		53

		38,991		0.089		0.04539		0.04361		49

		38,997		0.035		0.01715		0.01785		51

		39,006		0.4202		0.218504		0.201696		48

		39,009		0.07		0.0357		0.0343		49

		39,013		0.35		0.1855		0.1645		47

		39,019		0.0047		0.002632		0.002068		44

		39,024		0.1808		0.104864		0.075936		42

		39,038		0.035		0.021		0.014		40

		39,045		0.038		0.02318		0.01482		39

		39,049		0.025		0.01675		0.00825		33

		39,053		0.011		0.00759		0.00341		31

		39,059		0.075		0.0555		0.0195		26

				0.1006294118		0.0529635294				45.1176470588

		日期		进水





Sheet2

		

						TP

		日期		进水		出水		进水		去除浓度		去除率

		38,944		0.282		0.12126		0.282		0.16074		57

		38,954		0.014		0.00658		0.014		0.00742		53

		38,980		0.026		0.0117		0.026		0.0143		55

		38,983		0.041		0.0205		0.041		0.0205		50

		38,986		0.014		0.00658		0.014		0.00742		53

		38,991		0.089		0.04539		0.089		0.04361		49

		38,997		0.035		0.01715		0.035		0.01785		51

		39,006		0.4202		0.218504		0.4202		0.201696		48

		39,009		0.07		0.0357		0.07		0.0343		49

		39,013		0.35		0.1855		0.35		0.1645		47

		39,019		0.0047		0.002632		0.0047		0.002068		44

		39,024		0.1808		0.104864		0.1808		0.075936		42

		39,038		0.035		0.021		0.035		0.014		40

		39,045		0.038		0.02318		0.038		0.01482		39

		39,049		0.025		0.01675		0.025		0.00825		33

		39,053		0.011		0.00759		0.011		0.00341		31

		39,059		0.075		0.0555		0.075		0.0195		26

				0.1006294118		0.0529635294						45.1176470588





Sheet2

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0

		0



去除浓度

进水TP浓度 /mg.L-1

TP去除浓度/mg.L-1

y = 0.4941x - 0.0021

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0



Sheet3

		

				TP不同区段去除率的变化

				去除率

		4		16.64

		8		15.5

		12		7.41

		16		5.57

				45.12

				0





Sheet3

		0

		0

		0

		0



去除率

取样口位置/m

去除率/%

0

0

0

0



0


10


20


30


40


50


60


70


0.3 3.6 8.1 13 15 15.8


温度/


o


C


去除率/%


a b


c d





